IBM Spatial Idle-Correlation Preregistration

Corrected audit export

March 10, 2026

Presentation edition prepared from independently rerun scripts and live verification tests.

Date: 2026-03-08

## Question

Do idle-time error correlations on real IBM hardware follow an inverse-square law in graph distance more strongly than a local exponential decay law when measured on a fixed, preregistered spatial layout?

## Primary claim under test

The MSQECC hardware section predicts a nonlocal tail in idle-time correlations. Operationally, that means the pairwise absolute correlation |corr_ij| between measurement outcomes should be better described by

|corr_ij| = A / D_ij^2

than by

|corr_ij| = A exp(-D_ij / L)

where D_ij is the shortest-path distance on the device coupling graph.

## Backends

Primary backend: ibm_marrakesh

Replication backend: ibm_torino

These backends were selected before the new runs because they were available on the IBM open plan and have enough physical qubits to support a 10-qubit sparse layout.

## Fixed layouts

The layouts were chosen before running the new experiment by a deterministic maximin rule applied only to the backend coupling map: start from qubit 0, then iteratively add the qubit that maximizes the minimum graph distance to the already selected set, breaking ties by larger mean distance.

The resulting fixed layouts are:

ibm_marrakesh: 0,140,75,80,155,13,40,109,120,27

ibm_torino: 0,128,37,91,64,129,9,90,38,111

For the 10 selected qubits, this yields 45 pairwise distances per backend.

## Circuit

One register of 10 qubits.

No state preparation beyond the backend ground-state reset.

A single idle delay applied to all selected qubits.

Full measurement in the computational basis.

## Fixed run settings

Qubits per run: 10

Shots per run: 4000

Idle durations in microseconds: 0, 20, 50, 100, 200, 500

Same circuit family, same transpilation path, same analysis code for every run.

## Analysis plan

For each backend and idle duration:

Compute the 10x10 correlation matrix from raw measurement counts.

Extract the 45 off-diagonal absolute pair correlations |corr_ij|.

Fit two models to the pairwise dataset:

inverse-square: A / D^2

exponential: A exp(-D / L)

Compare models with BIC.

Convert Delta BIC to a large-sample approximate Bayes factor using BF = exp(Delta BIC / 2) in favor of the lower-BIC model.

## Preregistered decision rule

Evidence for the MSQECC-style nonlocal tail requires all of the following:

On ibm_marrakesh, the inverse-square model is preferred over the exponential model with approximate Bayes factor >= 10 in at least 3 of the 5 nonzero-delay runs.

On ibm_torino, the inverse-square model is preferred over the exponential model with approximate Bayes factor >= 10 in at least 2 of the 5 nonzero-delay runs.

The inverse-square preference must not be driven solely by the 0 us baseline.

If these conditions fail, the hardware test is recorded as negative or inconclusive for the claimed inverse-square signature.

## Interpretation guardrails

Graph distance, not Euclidean chip-coordinate distance, is the preregistered spatial metric because it is directly defined by the backend coupling graph and is stable across runtime metadata.

This experiment does not prove a vacuum theory even if positive. It tests only whether a specific distance-law signal is present in idle correlations.

A negative result counts against the hardware claim and should not be reframed as support.